The University of Azad Jammu and Kashmir, Muzaffarabad

| Name | Jawahir Ali |
| --- | --- |
| Course Name | Computer Architecture and Logic Design |
| Submitted to | Engr. Sidra Rafique |
| Semester | 2nd |
| Session | 2024-2028 |
| Roll No | 2024-SE-34 |
| Lab No | 03 |

Department of Software Engineering

# Procedure to Connect 7408 (AND Gate IC) in EWB

### 1. Open EWB Software

- Start the Electronic Workbench on your computer.

### 2. Place the 7408 IC

- Click on Place → Component.

- Go to the TTL or Digital library.

- Search and select 7408 (it’s the Quad 2-input AND gate).

- Place the 7408 IC onto the workspace.

### 3. Connect the Power Supply

- Pin 14: Connect it to +5V (power supply).

- Pin 7: Connect it to Ground (GND).

This powers up the IC so it works properly.

### 4. Set up Inputs

- From Inputs menu, pick Logic Switches.

- Place two switches (one for each input).

- Connect:

  - One switch to Pin 1 (Input A1).

  - One switch to Pin 2 (Input B1).

### 5. Set up Output

- From Indicators menu, place an LED.

- Put a 330Ω resistor in series with the LED (protects the LED).

- Connect:

  - Pin 3 (Output Y1) → Resistor → LED → Ground.

### 6. Start Simulation

- Click Run (or Start Simulation button).

### 7. Test the Circuit

- Use the switches to turn inputs ON or OFF.

- Observe the LED:

  - LED ON = Output is HIGH (1).

  - LED OFF = Output is LOW (0).

- Try all combinations (00, 01, 10, 11) and record the results.

AND GATE

Truth Table

Output

# Procedure to Connect 7432 (OR Gate IC) in EWB

### 1. Open EWB Software

- Start Electronic Workbench on your computer.

### 2. Place the 7432 IC

- Click Place → Component.

- Find the TTL or Digital family.

- Search for 7432 (Quad 2-input OR gates).

- Place the 7432 IC on your workspace.

### 3. Connect Power to the IC

- Pin 14: Connect to +5V (positive supply).

- Pin 7: Connect to Ground (GND).

⚡ This step powers the IC so it can work correctly during simulation.

### 4. Set Up Inputs

- From the Inputs menu, place two logic switches.

- Connect:

  - First switch to Pin 1 (Input A1).

  - Second switch to Pin 2 (Input B1).

### 5. Set Up Output

- From the Indicators menu, place an LED.

- Add a 330Ω resistor between the output and the LED.

- Connect:

  - Pin 3 (Output Y1) → Resistor → LED → Ground.

### 6. Start the Simulation

- Click Run to start the simulation.

### 7. Test and Observe

- Toggle the switches through different combinations (00, 01, 10, 11).

- Watch the LED:

  - LED ON = Output is HIGH (1).

  - LED OFF = Output is LOW (0).

- Record the output to make your truth table.

OR GATE

Truth table

Output

# Procedure to Connect 7400 (NAND Gate IC) in EWB

### 1. Open EWB Software

- Start Electronic Workbench on your computer.

### 2. Place the 7400 IC

- Go to Place → Component.

- Find the TTL or Digital library.

- Search for 7400 (Quad 2-input NAND gate).

- Drag and place the IC on your workspace.

### 3. Connect Power to the IC

- Pin 14: Connect it to +5V (positive supply).

- Pin 7: Connect it to Ground (GND).

⚡ This powers up the IC so it works properly in the simulation.

### 4. Setup Input Connections

- From the Inputs menu, place two logic switches.

- Connect:

  - First switch to Pin 1 (Input A1).

  - Second switch to Pin 2 (Input B1).

### 5. Setup Output Connection

- From the Indicators menu, place an LED.

- Add a 330Ω resistor in series with the LED.

- Connect:

  - Pin 3 (Output Y1) → Resistor → LED → Ground.

### 6. Start the Simulation

- Press the Run button to start the simulation.

### 7. Test and Observe

- Turn ON and OFF the switches in different combinations (00, 01, 10, 11).

- Watch the LED:

  - LED ON = Output is HIGH (1).

  - LED OFF = Output is LOW (0).

- Record the results for each combination to create the truth table.

NAND GATE

Truth table

Output

Practical Work in Lab

# Procedure to Connect 7402 (NOR Gate IC) in EWB

### 1. Open EWB Software

- Start the Electronic Workbench on your computer.

### 2. Place the 7402 IC

- Click Place → Component.

- Go to TTL or Digital library.

- Search for 7402 (Quad 2-input NOR gates).

- Place the 7402 IC onto your workspace.

### 3. Connect Power to the IC

- Pin 14: Connect to +5V (positive power supply).

- Pin 7: Connect to Ground (GND).

⚡ Powering the IC is necessary to make it work properly.

### 4. Set Up Inputs

- From the Inputs section, place two logic switches.

- Connect:

  - First switch to Pin 1 (Input A1).

  - Second switch to Pin 2 (Input B1).

### 5. Set Up Output

- From the Indicators section, place an LED.

- Add a 330Ω resistor between the output and LED.

- Connect:

  - Pin 3 (Output Y1) → Resistor → LED → Ground.

### 6. Start the Simulation

- Click Run to start the simulation.

### 7. Test and Observe

- Change the switches to different combinations (00, 01, 10, 11).

- Watch the LED:

  - LED ON = Output is HIGH (1).

  - LED OFF = Output is LOW (0).

- Write down the results to create the truth table.

NOR GATE

Truth table

Output

### 1. Open EWB Software

- Start Electronic Workbench on your computer

### 2. Place the 7404 IC

- Click on Place → Component.

- Find the TTL or Digital library.

- Search for 7404 (Hex Inverter/NOT gates).

- Place the 7404 IC onto your workspace.

### 3. Connect Power to the IC

- Pin 14: Connect to +5V (positive power).

- Pin 7: Connect to Ground (GND).

⚡ This step powers the IC so it works properly.

### 4. Set Up Input

- From the Inputs menu, place one logic switch.

- Connect the switch to Pin 1 (Input A1).

### 5. Set Up Output

- From the Indicators menu, place an LED.

- Add a 330Ω resistor between the output and LED.

- Connect:

  - Pin 2 (Output Y1) → Resistor → LED → Ground.

### 6. Start the Simulation

- Click Run to start simulation.

### 7. Test and Observe

- Turn the switch ON and OFF (0 or 1).

- Observe the LED:

  - If input is 0 → LED turns ON (Output 1).

  - If input is 1 → LED turns OFF (Output 0).

NOT GATE

Truth table

Output