Lab Report: Parity Generator and Checker with Shift Operations

University: The University of Azad Jammu and Kashmir Muzaffarabad
Name: Jawahir Ali
Roll No: 2024-SE-34
Course: Computer Architecture and Logic Design
Instructor: Engr. Sidra Rafique
Task: Open Ended Lab
Department: Software Engineering
Date: [Insert Date]

Table of Contents

- Introduction

- Objectives

- Equipment and Software Used

- Theoretical Background

- Implementation and Procedure

- Results and Analysis

- Discussion

- Conclusion

- References

1. Introduction

This lab focuses on designing and simulating digital circuits for error detection using parity bits and understanding low-level shift operations in assembly language. The two main tasks include:

- Designing a 4-bit even parity generator and checker.

- Implementing and testing shift instructions (SHL, SHR, SAR, SAL) using an emulator.

2. Objectives

- To design and simulate a 4-bit even parity generator and checker circuit.

- To verify the truth table for both generator and checker.

- To implement and test shift instructions in assembly language using emu8086.

3. Equipment and Software Used

- Digital logic simulator (e.g., Electronic Workbench, Logisim)

- emu8086 assembler and emulator

- Standard logic gates (XOR)

4. Theoretical Background

Parity Generator

A parity bit is used for error detection in data transmission. For even parity, the total number of 1’s (including the parity bit) must be even.

Boolean Equation for Parity Generator:

P=A⊕B⊕C⊕DP=A⊕B⊕C⊕D

Parity Checker

The checker verifies the integrity of the received data by comparing the parity bit.

Boolean Equation for Parity Checker:

E=A⊕B⊕C⊕D⊕PE=A⊕B⊕C⊕D⊕P

- E=0E=0: No error

- E=1E=1: Error detected

Shift Operations

- SHL: Shift Left Logical (multiply by 2)

- SHR: Shift Right Logical (divide unsigned number by 2)

- SAR: Shift Arithmetic Right (divide signed number by 2, preserve sign)

- SAL: Shift Arithmetic Left (same as SHL)

5. Implementation and Procedure

5.1 Parity Generator and Checker

- Designed using XOR gates in a digital logic simulator.

- Inputs: A, B, C, D (4-bit data word)

- Output: Parity bit P (for generator), Error signal E (for checker)

- Tested all 16 input combinations.

5.2 Shift Operations

- Written and executed in emu8086.

- Used instruction: MOV AL, 00001101b (load 13 in binary)

- Applied SHL, SHR, SAR, SAL and observed results.

Conclusion

The lab successfully demonstrated:

- The design and functionality of a 4-bit even parity generator and checker.

- The use of shift instructions for arithmetic operations in assembly language.

- The importance of parity bits in error detection and shift operations in low-level programming.